WRF-CA-10: Secondary Treatment Clarifiers R&R

| Project Duration | 2026 – 2030 (5 Years) |  |
| --- | --- | --- |
| Project Description | Replace the following assets as part of planned removal and replacement (R&R) due to asset end of useful life: Clarifiers 1-8 Mixers (MIX-008, MIX-009, MIX-010, MIX-011, MIX-012, MIX-013, MIX-014, MIX-015), North RAS Electrical Equipment (ELEC-004), North and South Clarifier Ejectors (PMP-014, PMP-015, PMP-016, PMP-017), South WAS Pumps 1 and 2 (PMP-020, PMP-021) |  |
| Existing Asset Information | - MIX-008/NEWRF-ST-C-N-C2-SAR: 0.75 HP ¦ 1690 RPM ¦ 230 V ¦ 2.5 A - MIX-009/NEWRF-ST-C-N-C1-SAR: 0.75 HP ¦ 1690 RPM ¦ 230 V ¦ 2.5 A - MIX-010/NEWRF-ST-C-N-C3-SAR: 0.75 HP ¦ 1690 RPM ¦ 230 V ¦ 2.5 A - MIX-011/NEWRF-ST-C-N-C4-SAR: 0.75 HP ¦ 1690 RPM ¦ 230 V ¦ 2.5 A - MIX-012/NEWRF-ST-C-S-C6-SAR: 0.75 HP ¦ 1710 RPM ¦ 270 V ¦ 1.35 A - MIX-015/NEWRF-ST-C-S-C7-SAR: 0.75 HP ¦ 1710 RPM ¦ 270 V ¦ 1.35 A - PMP-010/NEWRF-ST-RWPS-NPH-RASP1A-P: 1190 RPM ¦ 4500 GPM - PMP-011/NEWRF-ST-RWPS-NPH-RASP2A-P: 40 HP ¦ 1190 RPM ¦ 4500 GPM - ELEC-004: 40 HP ¦1190 RPM ¦ 460 V ¦ 46 A - PMP-014/NEWRF-ST-C-N-C12SPA-P: 35 GPM - PMP-015/NEWRF-ST-C-N-C34SPA-P: 35 GPM - PMP-016/NEWRF-ST-C-S-C56SPA-P: 35 GPM - PMP-017/NEWRF-ST-C-S-C78SPA-P: 35 GPM - PMP-020/NEWRF-ST-RWPS-SPH-WASP2A-P: 10 HP ¦ 200 GPM - PMP-021/NEWRF-ST-RWPS-SPH-WASP1A-P: 10 HP ¦ 200 GPM |  |
| Project Costs | Total Construction Costs | $596,100 |
|  | Scope Contingency Cost (60%) | $357,700 |
|  | Engineering Costs (20%) | $190,800 |
|  | Total Project Cost | $1,144,600 |
| Record Drawing References | - NORTHEAST WRF IMPROVEMENTS PROJECT, 2022, 19-0029-UT - CLEARWATER CLARIFIER REHABILITATION, 2010, 12-0025-UT |  |